防火墙虚拟墙旁挂实验

1实验设想

公司有四个部门，在sw1上分配两个vrf，将pc1和pc2的部门加入v1中，pc3和pc4加入v2中，分别进行互访需求，要求经过旁挂防火墙增加业务部门的安全性，同时避免公司内部人员攻击服务器。

2.拓扑设计

3.配置脚本

基础配置：

在SW1上创建两个vrf实例分别为V1和V2，防火墙上创建两个虚拟墙Vsys1和Vsys2

sysname SW1

#

vlan batch 10 20 30 40 50 60

#

ip vpn-instance v1  //创建实例v1

ipv4-family

route-distinguisher 1:1

#

ip vpn-instance v2  //创建实例v2

ipv4-family

route-distinguisher 2:2

#

interface Vlanif10

ip binding vpn-instance v1      //将vlan10划分到实例v1中

ip address 1.1.1.1 255.255.255.0

#

interface Vlanif20

ip binding vpn-instance v1   //将vlan20划分到实例v1中

ip address 2.2.2.2 255.255.255.0

#

interface Vlanif30

ip binding vpn-instance v2   //将vlan30划分到实例v2中

ip address 3.3.3.3 255.255.255.0

#

interface Vlanif40

ip binding vpn-instance v2   //将vlan40划分到实例v2中

ip address 4.4.4.4 255.255.255.0

#

interface Vlanif50            //sw1全局与fw1的vsys1互联网段

ip address 192.168.1.1 255.255.255.0

#

interface Vlanif60            //sw1全局与fw1的vsys2互联网段

ip address 192.168.2.1 255.255.255.0

#

interface GigabitEthernet0/0/1

port link-type access

port default vlan 10

#

interface GigabitEthernet0/0/2

port link-type access

port default vlan 20

#

interface GigabitEthernet0/0/3

port link-type access

port default vlan 30

#

interface GigabitEthernet0/0/4

port link-type access

port default vlan 40

#

interface GigabitEthernet0/0/5

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60

#

interface GigabitEthernet0/0/6

port link-type access

port default vlan 50   //server 网段

#

防火墙配置

Sys FW

vlan batch 10 20 30 40 50 60

vsys enable  //开启vsys虚拟墙功能

#

vsys name vsys1 1  //创建虚拟墙vsys1，并将vlan10 20 50划分到vsys1

assign vlan 10

assign vlan 20

assign vlan 50

#

vsys name vsys2 2  //创建虚拟墙vsys2，并将vlan30 40 60划分到vsys1

assign vlan 30

assign vlan 40

assign vlan 60

#

interface GigabitEthernet1/0/0

portswitch

undo shutdown

port link-type trunk

port trunk allow-pass vlan 10 20 30 40 50 60

#

interface Virtual-if0   //全局虚拟接口

ip address 172.16.0.3 255.255.255.0

#

firewall zone trust    //将全局的虚拟接口划分到trust区域

add interface Virtual-if0

#

switch vsys vsys1

interface Vlanif10

ip binding vpn-instance vsys1

ip address 1.1.1.253 255.255.255.0

service-manage ping permit

#

interface Vlanif20

ip binding vpn-instance vsys1

ip address 2.2.2.253 255.255.255.0

service-manage ping permit

#

interface Vlanif50

ip binding vpn-instance vsys1

ip address 192.168.1.253 255.255.255.0

service-manage ping permit

#

interface Virtual-if1   //vsys1虚拟接口

ip address 172.16.0.1 255.255.255.0

#

firewall zone trust

set priority 85

add interface Vlanif50

#

firewall zone untrust

set priority 5

add interface Virtual-if1

add interface Vlanif10

add interface Vlanif20

#

#

security-policy

rule name ttu

source-zone trust

destination-zone untrust

source-address 192.168.1.0 mask 255.255.255.0

service icmp

action permit

rule name utt

source-zone untrust

destination-zone trust

source-address 1.1.1.0 mask 255.255.255.0

source-address 2.2.2.0 mask 255.255.255.0

destination-address 192.168.1.0 mask 255.255.255.0

service icmp

action permit

rule name utu

source-zone untrust

destination-zone untrust

source-address 1.1.1.0 mask 255.255.255.0

source-address 2.2.2.0 mask 255.255.255.0

destination-address 3.3.3.0 mask 255.255.255.0

destination-address 4.4.4.0 mask 255.255.255.0

service icmp

action permit

switch vsys vsys2

#

interface Vlanif30

ip binding vpn-instance vsys2

ip address 3.3.3.253 255.255.255.0

service-manage ping permit

#

interface Vlanif40

ip binding vpn-instance vsys2

ip address 4.4.4.253 255.255.255.0

service-manage ping permit

#

interface Vlanif60

ip binding vpn-instance vsys2

ip address 192.168.2.253 255.255.255.0

service-manage ping permit

#

interface Virtual-if2   //vsys2虚拟接口

ip address 172.16.0.2 255.255.255.0

#

firewall zone trust

set priority 85

add interface Vlanif60

#

firewall zone untrust

set priority 5

add interface Virtual-if2

add interface Vlanif30

add interface Vlanif40

#

security-policy

rule name ttu

source-zone trust

destination-zone untrust

source-address 192.168.1.0 mask 255.255.255.0

service icmp

action permit

rule name utt

source-zone untrust

destination-zone trust

source-address 3.3.3.0 mask 255.255.255.0

source-address 4.4.4.0 mask 255.255.255.0

destination-address 192.168.1.0 mask 255.255.255.0

destination-address 192.168.2.0 mask 255.255.255.0

service icmp

action permit

rule name utu

source-zone untrust

destination-zone untrust

source-address 3.3.3.0 mask 255.255.255.0

source-address 4.4.4.0 mask 255.255.255.0

destination-address 1.1.1.0 mask 255.255.255.0

destination-address 2.2.2.0 mask 255.255.255.0

service icmp

action permit

需求一：

PC1访问PC3流量需求V1-Vsys1-全局-Vsys2-V2

交换机配置：

ip route-static vpn-instance v1 3.3.3.0 255.255.255.0 192.168.1.253

//sw1 去往pc3的路由，下一跳是防火墙的fw vsys1

ip route-static vpn-instance v1 192.168.1.0 255.255.255.0 1.1.1.253

//sw1 v1去往 fw vsys 1与全局互联的路由，下一跳是防火墙的v1 1.1.1.253

ip route-static vpn-instance v2 1.1.1.0 255.255.255.0 192.168.2.253

//sw1 v2去往1.1.1.1的路由下一跳是fw vsys2  （反向）

ip route-static 3.3.3.0 255.255.255.0 vpn-instance v2 192.168.2.253

//sw1全局去往pc3的路由，下一跳为fw vsys2

防火墙配置：

switch vsys1

ip route-static 3.3.3.0 255.255.255.0 192.168.1.1

//fw1 vsys1去往pc3网段的路由，下一跳是sw1 全局与fw vsys1互联地址

switch vsys2

ip route-static 1.1.1.0 255.255.255.0 192.168.2.1

//fw1 vsys去往pc1网段的路由，下一跳是sw1全局与fw vsys2互联地址（反向）

需求二：

PC2访问PC4流量要求V1-Vsys1-Vsys2-V2

ip route-static vpn-instance v1 4.4.4.0 255.255.255.0 2.2.2.253

//SW1 v1 去往pc4 4.4.4.100网段的路由，下一跳是FW2 vsys1

ip route-static vpn-instance vsys1 4.4.4.0 255.255.255.0 vpn-instance vsys2

//FW1 vsys1去往vsys2的路由

ip route-static vpn-instance vsys2 2.2.2.0 255.255.255.0 vpn-instance vsys1

//FW1 vsys2去往vsys1的路由

ip route-static vpn-instance v2 2.2.2.0 255.255.255.0 4.4.4.253

//SW1 v2 去往pc2 2.2.2.100网段的路由，下一跳是FW2 vsys2

需求三：

pc访问服务器，经过各自所在防火墙到全局

ip route-static 1.1.1.0 255.255.255.0 vpn-instance v1 192.168.1.253

//SW1 全局去往pc1的路由

ip route-static 2.2.2.0 255.255.255.0 vpn-instance v1 192.168.1.253

//SW1 全局去往pc2的路由

ip route-static 3.3.3.0 255.255.255.0 vpn-instance v2 192.168.2.253

//SW1 全局去往pc3的路由

ip route-static 4.4.4.0 255.255.255.0 vpn-instance v2 192.168.2.253

//SW1 全局去往pc4的路由

ip route-static vpn-instance v2 192.168.2.0 255.255.255.0 3.3.3.253

//SW1 v2去往全局的路由，下一跳是fw vsys2

防火墙配置：

ip route-static 192.168.1.0 255.255.255.0 192.168.2.1

//fw vsys2去往服务器网段的路由，下一跳为sw1全局与fw2 vsys2互联网段

4.总结

测试pc1与pc3互通以及路径

测试pc2与pc4互通以及路径

测试pc1，pc2，pc3，pc4与服务器互通以及路径

- tracert在测试路由所经过的路径时，如果经过防火墙则不会显示路径

- 防火墙记得加安全策略五元组，经常忘了配置服务的安全策略。一开始做的时候可以用any to any放通全部，等确保路由能够通信后再做安全策略

- 每个需要用到的接口都要划分zone，包括Virtual-if

- 需要测试ping的接口记得service-manage ping permit，防火墙主要先确保区域，策略，接口划分再去做路由，后面出错了就比较好排错。